RISC-V

Multi-Core Processor

ASIC Design Specification

| Document Type | Hardware Design Specification |
| --- | --- |

| Revision | 1.0 — Draft |
| --- | --- |

| ISA | RISC-V RV64GC (RV64IMAFDC) |
| --- | --- |

| Core Count | 4 (scalable to 8) |
| --- | --- |

| Target Technology | TSMC 28nm / 22nm FD-SOI |
| --- | --- |

| HDL Language | SystemVerilog (IEEE 1800-2017) |
| --- | --- |

| Synthesis Tool | Synopsys Design Compiler / Cadence Genus |
| --- | --- |

| Date | June 11, 2026 |
| --- | --- |

# 1. Introduction and Scope

This document defines the architectural design, microarchitecture, RTL implementation strategy, verification plan, and ASIC synthesis flow for a multi-core RISC-V processor implemented in SystemVerilog. The design targets an ASIC tape-out using standard-cell libraries and is intended for embedded computing, edge AI inference, or application-class workloads.

## 1.1 Design Goals

- Full compliance with the RISC-V RV64GC ISA (Integer, Multiply, Atomic, Float, Double, Compressed)

- 4-core homogeneous symmetric multiprocessing (SMP) with shared L2 cache

- In-order or lightly out-of-order pipeline (5–7 stages) per core

- AMBA AXI4 interconnect fabric for on-chip communication

- Coherent shared memory model (MESI protocol) across all cores

- Target PPA: >1 GHz @ 28nm, <500 mW TDP, <5 mm² die area

- Full synthesizability with zero latches; clean lint and CDC reports

## 1.2 Out of Scope

- Custom ISA extensions beyond standard RISC-V

- GPU or vector (V extension) units in v1.0

- PCB-level design, packaging, or physical layout (P&R)

- FPGA prototype (separate effort)

## 1.3 Reference Documents

| Document | Version | Source |
| --- | --- | --- |
| The RISC-V Instruction Set Manual Vol. I | 2.2 | riscv.org |
| The RISC-V Instruction Set Manual Vol. II (Privileged) | 1.12 | riscv.org |
| AMBA AXI4 Protocol Specification | E | developer.arm.com |
| AMBA ACE Protocol Specification (cache coherency) | D | developer.arm.com |
| IEEE Standard for SystemVerilog | 1800-2017 | IEEE |
| Synopsys Design Compiler User Guide | T-2022.03 | Synopsys SolvNet |
| TSMC 28HPM Design Rules | 1.4 | TSMC PDK (NDA) |

# 2. System Architecture

## 2.1 Top-Level Block Diagram

Block diagram description: Four RV64GC cores each with private L1-I and L1-D caches connect through a ring or crossbar interconnect to a unified L2 cache and main memory controller. Peripherals (UART, SPI, GPIO, Timer) hang off an APB bridge. An optional DMA engine and debug module (JTAG) complete the SoC periphery.

## 2.2 Core Complex

Each core is an independent RV64GC hart. Cores share no internal state but communicate via the cache-coherent interconnect. Each core instantiates:

- Instruction Fetch Unit (IFU) with branch predictor

- Instruction Decode Unit (IDU)

- Register File (32 × 64-bit integer + 32 × 64-bit FP)

- Execution Units: ALU, MUL/DIV, FPU, CSR unit

- Load/Store Unit (LSU) with store buffer

- L1 Instruction Cache: 32 KB, 4-way set-associative

- L1 Data Cache: 32 KB, 4-way set-associative, write-back/write-allocate

- L1 TLB: 32-entry fully associative (I-TLB and D-TLB)

## 2.3 Shared Subsystems

| Subsystem | Specification | Interface |
| --- | --- | --- |
| L2 Cache | 512 KB unified, 8-way SA, MESI coherence | AMBA ACE (128-bit) |
| Memory Controller | LPDDR4 / AXI DRAM, ECC optional | AXI4 Master (256-bit) |
| Interrupt Controller | RISC-V PLIC, 64 sources, 8 priorities | AXI4-Lite Slave |
| Debug Module | RISC-V Debug Spec 0.13.2, JTAG TAP | DMI (Debug Module Interface) |
| Boot ROM | 16 KB, hardened, JTAG programmable | AXI4-Lite Slave |
| DMA Controller | 4-channel, scatter-gather, AXI4 burst | AXI4 Master + Slave |
| Clock/Reset | PLL × 2, async reset sync, scan enable | Custom CSRs |

## 2.4 Memory Map

| Base Address | Size | Region | Attributes |
| --- | --- | --- | --- |
| 0x0000_0000 | 16 KB | Boot ROM | R-X, cached |
| 0x0200_0000 | 64 KB | CLINT (timer/sw IRQ) | RW, MMIO |
| 0x0C00_0000 | 4 MB | PLIC | RW, MMIO |
| 0x1000_0000 | 4 KB | UART0 | RW, MMIO |
| 0x1000_1000 | 4 KB | SPI0 | RW, MMIO |
| 0x2000_0000 | 256 MB | Internal SRAM (optional) | RWX, cached |
| 0x8000_0000 | 2 GB | DRAM (via memory controller) | RWX, cached |
| 0xF000_0000 | 256 MB | Debug / Test MMIO | RW, MMIO |

# 3. Core Microarchitecture

## 3.1 Pipeline Overview

Each RV64GC core implements a 7-stage in-order pipeline with a single issue slot. The stages are:

| Stage | Abbreviation | Description |
| --- | --- | --- |
| Fetch 1 | F1 | PC mux, branch predictor lookup, I-cache tag check |
| Fetch 2 | F2 | I-cache data read, instruction alignment, 16/32-bit decode |
| Decode | ID | Full instruction decode, register rename (in-order), hazard detect |
| Issue / Dispatch | IS | Operand forwarding, stall insertion, CSR access check |
| Execute | EX | ALU / MUL / FPU / LSU address calc (one cycle for most ops) |
| Memory | MEM | D-cache access, store buffer drain, TLB walk on miss |
| Write-back | WB | Register file write, exception commit, PC redirect on mispredict |

## 3.2 Branch Prediction

- 2-bit saturating counter bimodal predictor: 512-entry BHT (default)

- 16-entry return address stack (RAS) for JALR/JAL return pairs

- 64-entry BTB (Branch Target Buffer) for indirect jump targets

- Misprediction penalty: 4 cycles; flush from EX stage

- Upgrade path: TAGE predictor in v1.1 using same interface

## 3.3 Instruction Fetch and Compressed Extension

The IFU fetches a 64-bit (8-byte) aligned chunk per cycle, splitting it into up to two RVC (16-bit) or one RVI (32-bit) instruction. A 4-entry fetch buffer decouples the fetch and decode stages, absorbing I-cache miss stalls.

## 3.4 Execution Units

| Unit | Operations | Latency (cycles) | Pipeline Stages |
| --- | --- | --- | --- |
| Integer ALU | ADD/SUB/AND/OR/XOR/SLT/shifts/LUI/AUIPC | 1 | EX |
| Multiplier | MUL/MULH/MULW (RISC-V M ext.) | 3 (pipelined) | EX×3 |
| Divider | DIV/DIVU/REM/REMU/W variants | 32 (iterative) | EX (stall) |
| FPU (DP) | FADD/FMUL/FDIV/FSQRT/FMA, IEEE 754-2008 | 4 / 12 / 20 | EX–WB |
| CSR Unit | CSRRW/RS/RC, privilege checks | 1 (serialized) | IS/EX |
| LSU | LOAD/STORE/AMO/LR/SC (A ext.) | 1 (cache hit) | EX/MEM |

## 3.5 Load/Store Unit and Store Buffer

- 4-entry store buffer with address/data CAM for load-to-store forwarding

- Non-blocking loads: up to 1 outstanding miss with MSHR

- Misaligned access handled in hardware (sub-word granularity)

- AMO (atomic) operations serialized via cache lock protocol

## 3.6 Privilege Levels and CSRs

- Machine (M), Supervisor (S), and User (U) privilege levels

- Full Sv39 (39-bit virtual addressing) page table walker in hardware

- Physical Memory Protection (PMP): 16 entries per core

- MTIME, MTIMECMP, MSIP via CLINT MMIO (not per-core CSR)

- Performance counters: 8 × 64-bit HPM counters per core

# 4. Cache Hierarchy and Coherency

## 4.1 L1 Cache Design

Both I-cache and D-cache use physically indexed, physically tagged (PIPT) addressing to avoid aliasing. Cache line size is 64 bytes (512 bits). Replacement policy is pseudo-LRU (PLRU).

| Parameter | L1-I Cache | L1-D Cache |
| --- | --- | --- |
| Size | 32 KB | 32 KB |
| Associativity | 4-way SA | 4-way SA |
| Line size | 64 bytes | 64 bytes |
| Sets | 128 | 128 |
| Replacement | PLRU | PLRU |
| Write policy | Read-only (instruction) | Write-back / Write-allocate |
| ECC | Parity (SECDED optional) | SECDED |
| Ports | 1 read (64-bit) | 1 read + 1 write (64-bit each) |

## 4.2 L2 Cache and Coherence

The unified L2 cache acts as the Last Level Cache (LLC) for all four cores. It implements the MESI (Modified/Exclusive/Shared/Invalid) directory-based protocol. The L2 maintains a directory tracking which L1 caches hold each line.

- L2 size: 512 KB (configurable to 1 MB), 8-way set-associative

- L2 line size: 64 bytes (same as L1 to simplify inclusion policy)

- Inclusive policy: all L1 lines are present in L2

- Coherence messages: GetS, GetM, PutS, PutM, Inv, FwdGetS, FwdGetM, Ack

- Snoop filter: 4-core directory, 1 presence bit per core per line

## 4.3 Cache Coherence State Machine (MESI)

Per-cache-line state transitions: Invalid → Shared (read hit from memory), Shared → Invalid (invalidation on foreign write), Shared → Modified (upgrade via GetM), Modified → Invalid (eviction writeback), Exclusive → Modified (silent local write).

## 4.4 Interconnect

The on-chip network uses a 2D ring topology connecting the four core complexes and the L2 cache. At 4 cores, a ring provides acceptable latency (~10 cycles core-to-L2) with minimal area overhead. The ring operates at full core frequency with 128-bit flit width.

- Protocol: ACE (AXI Coherency Extensions) over 128-bit channels

- Ring stop per core + L2 + memory controller + DMA

- Flow control: credit-based to prevent deadlock

- Upgrade path to crossbar at 8+ cores

# 5. RTL Implementation (SystemVerilog)

## 5.1 File and Module Hierarchy

The following table shows the top-level module decomposition. Each module maps to a single .sv file. All parameterization is done via SystemVerilog parameters (not `define macros) to allow per-instantiation override.

| Module | File | Description |
| --- | --- | --- |
| riscv_soc_top | riscv_soc_top.sv | SoC top-level; instantiates all subsystems |
| riscv_core_complex | riscv_core_complex.sv | 4× core + L1 caches + ACE port |
| riscv_core | riscv_core.sv | Single RV64GC hart; instantiates pipeline stages |
| riscv_ifu | riscv_ifu.sv | Instruction fetch, branch predictor, I-cache ctrl |
| riscv_idu | riscv_idu.sv | Decode, register file read, hazard detection |
| riscv_exu | riscv_exu.sv | Execute: ALU, MUL/DIV, FPU, CSR |
| riscv_lsu | riscv_lsu.sv | Load/store, store buffer, D-cache ctrl, AMO |
| riscv_l1_icache | riscv_l1_icache.sv | 32 KB 4-way I-cache with PLRU |
| riscv_l1_dcache | riscv_l1_dcache.sv | 32 KB 4-way D-cache with MESI L1 side |
| riscv_tlb | riscv_tlb.sv | 32-entry FA TLB, Sv39 PTW |
| riscv_l2_cache | riscv_l2_cache.sv | 512 KB L2 with directory and ring interface |
| riscv_ring_noc | riscv_ring_noc.sv | 4-stop ring NoC, ACE protocol |
| riscv_plic | riscv_plic.sv | Platform Level Interrupt Controller |
| riscv_clint | riscv_clint.sv | Core Local Interruptor (timer + SW IRQ) |
| riscv_debug_module | riscv_debug_module.sv | RISC-V debug spec 0.13, JTAG TAP |
| riscv_dma | riscv_dma.sv | 4-channel scatter/gather DMA |
| riscv_uart | riscv_uart.sv | 16550-compatible UART |
| riscv_clkrst | riscv_clkrst.sv | PLL wrapper, clock gating, reset sync |

## 5.2 Coding Guidelines

### 5.2.1 General Synthesizability Rules

- All flip-flops use synchronous active-high reset unless required otherwise (scan DFT compliance)

- No latches — all state held in always_ff blocks; use always_comb for combinational logic

- No initial blocks in synthesizable RTL (use reset for initialization)

- No delays (#X), force/release, or $display in synthesis-targeted files

- All module ports explicitly typed; no implicit nets

- Use logic (not reg or wire) throughout

### 5.2.2 Clocking and Reset

- Single primary clock domain: clk_core (1 GHz target)

- Secondary domain: clk_apb (250 MHz) for slow peripherals; cross via 2FF synchronizer

- Clock gating: use ICG (Integrated Clock Gate) cells from PDK; never gate with combinational logic

- Async reset synchronizer on every domain: assert async, deassert synchronously

### 5.2.3 Parameterization

All configurable parameters must be declared at the module level:

module riscv_core #(

parameter int unsigned XLEN      = 64,  // 32 or 64

parameter int unsigned ILEN      = 32,  // max instruction width

parameter int unsigned NUM_HARTS = 1,   // harts per core module

parameter int unsigned L1I_SIZE  = 32*1024,

parameter int unsigned L1D_SIZE  = 32*1024,

parameter bit          FPU_EN    = 1'b1,

parameter bit          RVC_EN    = 1'b1

) (

input  logic        clk, rst_n,

// ... ACE master port, debug, IRQ ...

);

### 5.2.4 Interface Definitions

All inter-module connections with more than 4 signals must use a SystemVerilog interface. Key interfaces:

- riscv_pipe_if: inter-stage pipeline registers (valid, data, control)

- riscv_ace_if: ACE master/slave port (parameterized data width)

- riscv_apb_if: APB4 peripheral interface

- riscv_jtag_if: TCK, TMS, TDI, TDO, TRST_N

## 5.3 Key RTL Snippets

### 5.3.1 Pipeline Stage Register Template

// EX/MEM pipeline register

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

ex_mem_q <= '0;

end else if (flush_ex) begin

ex_mem_q <= '0;  // bubble on misprediction

end else if (!stall_mem) begin

ex_mem_q.valid   <= ex_valid;

ex_mem_q.rd      <= ex_rd;

ex_mem_q.alu_out <= alu_result;

ex_mem_q.mem_we  <= ex_mem_write;

ex_mem_q.store_d <= ex_store_data;

end

end

### 5.3.2 MESI Cache Controller FSM Skeleton

typedef enum logic [2:0] {

MESI_INVALID   = 3'b000,

MESI_SHARED    = 3'b001,

MESI_EXCLUSIVE = 3'b010,

MESI_MODIFIED  = 3'b011,

MESI_PENDING   = 3'b100   // awaiting coherence ack

} mesi_state_e;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) mesi_state[idx] <= MESI_INVALID;

else case (mesi_state[idx])

MESI_INVALID:  if (cpu_read)  mesi_state[idx] <= MESI_PENDING;

MESI_PENDING:  if (mem_ack && !shared) mesi_state[idx] <= MESI_EXCLUSIVE;

else if (mem_ack &&  shared) mesi_state[idx] <= MESI_SHARED;

MESI_SHARED:   if (cpu_write) mesi_state[idx] <= MESI_PENDING; // GetM

else if (inv_req)  mesi_state[idx] <= MESI_INVALID;

MESI_EXCLUSIVE:if (cpu_write) mesi_state[idx] <= MESI_MODIFIED;

MESI_MODIFIED: if (evict)     mesi_state[idx] <= MESI_INVALID; // writeback

endcase

end

# 6. Verification Strategy

## 6.1 Testbench Architecture

The verification environment follows the UVM (Universal Verification Methodology) framework with the following components:

- riscv_env: top-level UVM environment containing all agents and scoreboards

- riscv_isa_agent: drives random instruction streams; functional coverage of all opcodes

- riscv_mem_agent: models memory, injects latency and errors

- riscv_irq_agent: randomizes interrupt timing and priority

- riscv_jtag_agent: drives JTAG debug sequences

- riscv_scoreboard: golden reference model comparison (ISS-based)

- riscv_coverage: functional coverage groups for pipeline hazards, CSR states, cache states

## 6.2 Test Plan

| Test Suite | Target | Pass Criteria |
| --- | --- | --- |
| RISC-V Compliance Tests (riscv-arch-test) | All RV64IMAFDC instructions | 100% pass |
| riscv-tests (isa/ + mt/) | Privileged spec, machine/supervisor/user modes | All tests pass |
| Custom directed tests | Pipeline hazards: RAW, WAW, structural, control | 0 mismatches vs ISS |
| Cache coherence directed | MESI transitions, false sharing, upgrade | Protocol checker clean |
| Multi-core random (SMP stress) | 4-core shared memory with atomic ops | Litmus tests pass |
| Interrupt/exception directed | PLIC arbitration, trap entry/exit, mret | Precise exceptions |
| Debug module tests | Halt, single-step, breakpoint, memory access via JTAG | Full coverage |
| Power regression | Toggle coverage ≥ 90%, CDC assertions clean | No glitch reports |
| Gate-level simulation (post-synthesis) | Key directed tests on netlist + SDF | Timing clean |

## 6.3 Formal Verification

- Property checking on cache coherence protocol using JasperGold (or SymbiYosys for open-source flow)

- Liveness properties: every GetS must eventually be satisfied

- Safety properties: no two caches hold a line in Modified state simultaneously

- Pipeline properties: forwarding eliminates all RAW hazards; no starvation under any input

## 6.4 Instruction Set Simulator (ISS) Reference

Spike (the official RISC-V ISS) is used as the golden reference model. A co-simulation harness runs Spike and the RTL DUT in lockstep, comparing committed instruction results, register writes, memory writes, and exception causes at every retirement.

# 7. Synthesis and ASIC Implementation Flow

## 7.1 EDA Toolchain

| Step | Tool (Commercial) | Tool (Open Source) | Output |
| --- | --- | --- | --- |
| RTL Lint | Synopsys SpyGlass | Verilator --lint-only | Lint-clean RTL |
| CDC Analysis | Synopsys SpyGlass CDC | none | CDC waiver list |
| Logic Synthesis | Synopsys Design Compiler / Genus | Yosys (stdcell) | Gate-level netlist (.v) |
| Formal Equiv. | Synopsys Formality | Yosys equiv | LEC clean |
| STA | Synopsys PrimeTime | OpenSTA | Timing report, SDC |
| P&R | Cadence Innovus / Synopsys ICC2 | OpenROAD | GDS-II |
| DRC/LVS | Mentor Calibre | Magic + Netgen | Clean sign-off |
| Power Analysis | Synopsys PrimeTime PX | OpenROAD pdngen | Power report |

## 7.2 Synthesis Constraints (SDC Skeleton)

# Primary clock

create_clock -period 1.0 -name clk_core [get_ports clk_core]

create_clock -period 4.0 -name clk_apb  [get_ports clk_apb]

# Clock uncertainty and transition

set_clock_uncertainty 0.05 [get_clocks clk_core]

set_clock_transition  0.05 [get_clocks clk_core]

# I/O delays (relative to pad ring, 20% of period)

set_input_delay  0.2 -clock clk_core [all_inputs]

set_output_delay 0.2 -clock clk_core [all_outputs]

# False paths across async clock domains

set_false_path -from [get_clocks clk_apb] -to [get_clocks clk_core]

# Area and power targets

set_max_area 0

set_max_dynamic_power 500 mW

## 7.3 Design-for-Test (DFT)

- Scan insertion: full-scan with mux-scan flip-flops (PDK DFT cells)

- Scan chains: 8 chains per core (balanced for scan shift frequency 50 MHz)

- ATPG: Synopsys TetraMAX targeting ≥99% stuck-at and transition fault coverage

- MBIST: built-in self-test for all SRAM macros (L1/L2 cache, TLBs, register files)

- Boundary scan: JTAG IEEE 1149.1 at top-level pads

## 7.4 Power Management

- Clock gating: fine-grain ICG on each pipeline stage enable; coarse-grain on FPU and D-cache

- Power domains: core complex in one domain, peripherals in separate always-on domain

- Retention registers on CPU core for deep-sleep resume (optional)

- Dynamic voltage and frequency scaling (DVFS): interface exposed via CSRs to firmware

## 7.5 Target PPA (28nm)

| Metric | Target | Stretch Goal |
| --- | --- | --- |
| Frequency | 1.0 GHz (TT/0.9V/25°C) | 1.2 GHz |
| Core area (single core + L1) | 0.8 mm² | 0.6 mm² |
| Total die area (4-core + L2 + misc) | 4.5 mm² | 3.5 mm² |
| Dynamic power (4 cores active) | 450 mW @ 1 GHz | 350 mW |
| Leakage power | <20 mW | <15 mW |
| Dhrystone MIPS/core | ≥1200 DMIPS | ≥1500 DMIPS |

# 8. Development Phases and Milestones

| Phase | Duration | Deliverables | Exit Criteria |
| --- | --- | --- | --- |
| Phase 1: Single-Core RTL | 8 weeks | riscv_core, IFU, IDU, EXU, LSU, L1 caches | Passes RISC-V arch-test suite; 0 lint errors |
| Phase 2: Privilege + Debug | 4 weeks | CSR unit, PTW (Sv39), PLIC, CLINT, debug module | Runs Linux boot on ISS co-sim |
| Phase 3: Multi-Core + Coherence | 8 weeks | L2 cache, ring NoC, MESI protocol, SMP tests | All litmus tests pass; formal property check clean |
| Phase 4: SoC Integration | 4 weeks | UART, SPI, DMA, memory controller, SoC top | Integration regression passes |
| Phase 5: Synthesis + STA | 4 weeks | Netlist, SDC, STA reports | Timing clean at 1 GHz; gate-level sim passes |
| Phase 6: DFT + Sign-off | 4 weeks | Scan netlist, MBIST, power report, GDS hand-off | ≥99% test coverage; DRC/LVS clean |

# 9. Risks and Mitigations

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Cache coherence protocol deadlock | Medium | High | Formal verification with JasperGold; directed stress tests |
| Timing closure at 1 GHz | Medium | High | Early floorplan; pipeline retiming; reduce critical path in MUL/DIV |
| SRAM macro unavailability | Low | High | Use foundry SRAM compiler; register-file fallback for small arrays |
| FPU area/timing overhead | Medium | Medium | License or hard-macro FPU; disable via parameter if needed |
| Multi-clock CDC failures | Low | High | SpyGlass CDC from day 1; 2FF synchronizers on all crossings |
| ISA compliance gaps | Low | Medium | Run riscv-arch-test in CI on every RTL commit |

# Appendix A: Glossary

| Term | Definition |
| --- | --- |
| ACE | AMBA Coherency Extensions — cache-coherent extension to AXI |
| AMO | Atomic Memory Operation (RISC-V A extension: LR/SC, AMOSWAP, etc.) |
| ATPG | Automatic Test Pattern Generation |
| BTB | Branch Target Buffer — caches predicted target addresses |
| DFT | Design for Testability — scan, MBIST, boundary scan |
| DXA | Device-independent twip (1/1440 inch) — unit used in DOCX/DXA |
| FPU | Floating-Point Unit |
| ICG | Integrated Clock Gating cell |
| ISS | Instruction Set Simulator (e.g., Spike for RISC-V) |
| LEC | Logic Equivalence Checking — formal proof that synthesis preserved logic |
| LLC | Last Level Cache |
| LSU | Load/Store Unit |
| MBIST | Memory Built-In Self-Test |
| MESI | Modified / Exclusive / Shared / Invalid cache coherence protocol |
| NoC | Network on Chip |
| PPA | Power, Performance, Area — the three key ASIC metrics |
| PLRU | Pseudo Least Recently Used — cache replacement policy |
| PTW | Page Table Walker (hardware) |
| RAS | Return Address Stack |
| SDF | Standard Delay Format — annotates post-layout timing to simulation |
| SMP | Symmetric Multiprocessing |
| STA | Static Timing Analysis |
| TLB | Translation Lookaside Buffer |
| UVM | Universal Verification Methodology |

# Appendix B: Recommended Open-Source Tools

| Tool | Purpose | URL |
| --- | --- | --- |
| Verilator | Fast RTL simulation, lint checking | verilator.org |
| Spike (riscv-isa-sim) | Official RISC-V ISS / golden reference | github.com/riscv-software-src/riscv-isa-sim |
| riscv-arch-test | ISA compliance test suite | github.com/riscv-non-isa/riscv-arch-test |
| riscv-tests | Low-level ISA and privilege tests | github.com/riscv-software-src/riscv-tests |
| Yosys | Open-source synthesis | github.com/YosysHQ/yosys |
| OpenROAD | Open-source P&R flow | theopenroadproject.org |
| SymbiYosys | Formal verification front-end | github.com/YosysHQ/sby |
| OpenSTA | Open-source STA engine | github.com/The-OpenROAD-Project/OpenSTA |
| GTKWave | Waveform viewer | gtkwave.sourceforge.net |
| CocoTB | Python-based testbench framework | cocotb.org |